CS226- Hardware Lab  8

Hardware design using HDL(Verilog)

The goal of this lab is to familiarize the students with describing  computer architectural blocks in Verilog Hardware Description Language (HDL) and to learn how to handle the  simulator. The goal is also to  interpret outputs from a simulator and to understand how Verilog code is interpreted by the simulator.  Further we will simulate various computer harware building blocks.

Simulator  (modelsim).

Install Modelsim: http://model.com/content/modelsim-pe-student-edition-hdl-simulation

In windows, create a lab directory for the course in your home directory (eg. c:/cs225/Lab8). Download the  Lab8 files .

Start modelsim

Start -> programes->modelsim->

Create a project (File->New->Project)

Name the project and project location as shown

Figure 1

Task1: simulating an AND gate

In this exercise you shall simulate an AND gate (figure 2). Consider the code in listing. This code declares a single module, which you can think of as a class. The module's name is and_gate, which is descriptive, as we want to simulate an AND gate.

Figure 2

Figure 3

// name: and_gate.v

module and_gate(out, a,b);  //you list all inputs and outputs, by convention outputs go first

output out;                          // this tells the compile which lines are inputs and outputs

input a, b;

assign out = a & b;         // output function

endmodule

module tb_and_gate();   // Test bench fortb_ and_gate.v

reg a,b;                         // a reg, to allow us to assign the input, and a wire to receive the output

wire out;

and_gate uut(out,a,b); // this instantiates a and gate, uut is a label

initial

begin

a = 1'b0;                   // here we apply inputs to the gate

b = 1'b0;

#10;

a = 1'b0;

b = 1'b1;

#10;

a = 1'b1;

b = 1'b1;

#10;

a = 1'b1;

b = 1'b0;

#10;

end

// set up the monitoring

initial

begin

$monitor("a=%b, b=%b, out=%b, time=%t\n", a, b, out, $time);

end

endmodule

Now that we have a module (and_gate), we need to make an instance and simulate it for testing purposes. We only need to simulate and test, so we need to write a test bench. The terminology goes back to the old days of hardwired testing (usually with wire wrap), where you would have a physical testing bench, which typically was set up to rapidly connect and test circuits. The test bench creates virtual environment to verify the correctness   design (in this case “and” gate).  See figure 3 for a conceptual representation.

The $monitor command continuously monitors the values of the variables or signals specified in the parameter list and displays all parameters in the list whenever the values of any one variable or signal changes. $monitor only needs to be invoked once. Only one monitor list can be active at a time.

Step by step intructions:

From modelsim winow: File-> Add to Project->Existing Files: Select file ‘and_gate.v’ and tb_and_gate.v

Select and_gate.v in the project tab

Compile

Now compile tb_and_gate.v

To simulate:

Simulate -> Start  simulation

Click the plus sign (‘+’)  near work

Choose tb_and_gate and click OK

In the structural window (labeled as ‘sim’) you can see the structure of the design. The source code for the module that is chosen  in the structure window is displayed in the source window.

In the object window: Right click and choose Add to Wave -> signals in Region

To simulate select; Simulation -> Run -> Run -all

In the wave window, you can see the waveform for the selected signals

In this simulation outputs are also visible at main console window.

Study the simulation output and compare them  with the code. Examine the output signal of the gate.

Fig4: Simulation output

Task2: Repat the above step by creating  or_gate.v and tb_or_gate.v

// name: or_gate

module or_gate(out,a,b);  //you list all inputs and outputs, by convention outputs go first

output out;                          // this tells the compile which lines are inputs and outputs

input a, b;

assign out = a | b;         // output function

endmodule

module tb_or_gate();   // Test bench for and_gate.v

reg a,b;                         // a reg, to allow us to assign the input, and a wire to receive the output

wire out;

or_gate uut (out,a,b); // this instantiates a and gate, uut is a label

initial

begin

a = 1'b0;                   // here we apply inputs to the gate

b = 1'b0;

#10;

a = 1'b0;

b = 1'b1;

#10;

a = 1'b1;

b = 1'b1;

#10;

a = 1'b1;

b = 1'b0;

#10;

end

// set up the monitoring

initial

begin

$monitor("a=%b, b=%b, out=%b, time=%t\n", a, b, out, $time);

end

endmodule.

Exercise:  Simulate 3input /  input AND gate and OR gate. Write appropriate testbench.

Task 3: Types Modeling  in Verilog

In this part we will model  a logic function in different ways. Consider the following table

| sel | In2 | In1 | out |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

Out put can be simplied as

Out=in1. sel +in2. sel’ and the implementation is shown in figure  4

Figure 4

There are various ways of modeling  in Verilog.

- Structural

- Behavioural level

Let us call this function as mux and now we model the function in the above 3 methods.

- Structural

Structural Style: The circuit is specified in terms of lower level components ( in this example logic gates, which are Verilog primitives) connected with internal signals. The translation of such a specification into a physical circuit is straight forward.

// lines start with “//” is a comment

// name: mux_gate.v   // gate level model

module mux_struct(out, in1,in2,sel );  //you list all inputs and outputs, by convention outputs go first

output out;                          // this tells the compile which lines are inputs and outputs

input in1,in2,sel;

and a1 (a1_o,in1,sel);         // defines the a1 gate, see figure

not n1 (inv_sel,sel);         // defines the inverter gate, see figure

and a2 (a2_o,in2,inv_sel);         // defines the a2 gate, see figure

or  o1 (out, a1_o, a2_o);         // defines the 01 gate, see figure

endmodule

- Behavioural level

- Behavioural Style: It specifies the circuit in terms of its expected behaviour. It is the closest to a natural language description of the circuit functionality.

// lines start with “//” is a comment

// name: mux_behav.v   // Behavior: event-driven behavior description construct

module mux_behav(out, in1,in2,sel );  //you list all inputs and outputs, by convention outputs go first

output out;                          // this tells the compile which lines are inputs and outputs

input in1,in2,sel;

reg out;

always@(in1 or in2 or sel)

begin

if (sel)

out =in1;

else

out=in2;

end

endmodule

- Behavioural Style:

// lines start with “//” is a comment

module mux_ behav2(out, in1,in2,sel );  //you list all inputs and outputs, by convention outputs go first

output out;                          // this tells the compile which lines are inputs and outputs

input in1,in2,sel;

assign out=sel ? in1: in2;

endmodule

module tb_mux_behav();

reg in1,in2,sel;

wire out;

mux_behav UUT (out,in1,in2,sel);

initial

begin

in1 = 1'b0;                   // here we apply inputs to the logic

in2 = 1'b0;

sel = 1'b0;

#10;

in1 = 1'b0;

in2 = 1'b1;

sel = 1'b0;

#10;

in1 = 1'b1;

in2 = 1'b0;

sel = 1'b0;

#10;   end

// set up the monitoring

initial  begin

$monitor("sel=%b, in1=%b, in2=%b, out=%b, time=%t\n", sel, in1,in2, out, $time); end

endmodule

Your  Lab Assignment 8 (60 points, 10 points each)

Submission Your submission must contain:

- The source code of your design/testbench if any (for each of the problem) - reasonably well documented.

- A minimal document detailing the  design/ testbench/answers.

Course work submission  through cs225.iitp@gmail.com  with subject:  YourrollNo_Lab8.

Due at   end of the Lab session.

- Model the following logic function using Verilog gate level description and design appropriate test bench.

- A car has a fuel level detector that outputs the current fuel level as a 3-bit binary number, with 000 meaning empty and 111 meaning full. Create a simplified logic circuit (use Boolean algebra for simplification) that illuminates a  “low fuel” indicator light (by setting an output L to 1) when the fuel level drops below level 3.  Develop a Verilog model  for the logic and  test  bench to verify the model.

- Use a simulation to demonstrate whether or not the following functions are equal.  (Be sure to state which, if any, are equal.)  and verify using truth table

f  =  a b'  +  b' c'  +  a c

g  =  (b' + c) (a + b + c')

h  =  b' c'  +  b c  +  a c

- Simulate the following function using appropriate test bench.

module comparators(eq,neq,lt,lte,gt,gte,a,b);

parameter N = 8;

input  [N-1:0] a, b;

output         eq, neq;

output         lt, lte;

output         gt, gte;

assign eq  = (a == b);

assign neq = (a != b);

assign lt  = (a < b);

assign lte = (a <= b);

assign gt  = (a > b);

assign gte = (a >= b);

endmodule

(5)  A museum has three rooms, each with a motion sensor (m0, m1, m2) that outputs 1 when motion is detected. At night, the only person in the museum is one security guard who walks from room to room. Create a circuit that sounds an alarm (by setting an output A to 1) if motion is ever detected in more than one room at a time (i.e., in two or three rooms), meaning there must be an intruder or intruders in the museum.    Develop a Verilog model  for the above and  write a test bench to verify the model.

(6)Write a structural description of the following  schematic. Check the functionality of the circuit by writing appropriate test bench.

(6)Write a structural description of the following  schematic. Check the functionality of the circuit by writing appropriate test bench.

(7) Design 4 to 1 multiplexer  with different coding styles ( at lease 5) and test using appropriate test bench. Include all the designs in a single file( p7.v and tb_P7.v) and  submit.